|  | 41/11 = 3,73 | 41/10 = 4,10 | 41/9 = 4,56 | 37/8 = 4,63 | 41/8 = 5,13 |
| --- | --- | --- | --- | --- | --- |
| Dana 60 |  | Silverado MWM Sprint |  |  |  |
| Dana 70 HD | VW 6.110/7.110 (MWM Turbo – 89 ... | Agrale 7000D F-4000 VW 6.90/7.90 (79-96 MWM | Agrale 7000DX F-4000 (MWM 229) VW 6.90/7.90 |  |  |
| Dana 80 |  |  | Agrale 7500TDX Puma 7900 | F-Cargo 814 (cummins) F-4000 (freio a disco) VW 8.140 | Agrale 7500MA |

| Dana 60-F Application: 2500 and 3500 4x4 regular cab, 2500HD and 3500 4x4 Club Cab (with NV241 and NV241HD transfer case) Capacity: 4,850 lbs Type: Hypoid Ratios avail 3.54,4.10 Ring gear Diameter 9.75" Pinion mount Overhung Pinion shaft diameter 1.63" inner bearing 1.25" outer bearing Differential Pinions Two Axle Shaft material Low carbon Alloy Axle shaft diameter 1.31" inner 1.34" outer axle spline pitch 1.5" Spring Centers 42.15" Universal Single Cardian Turning Angle 36 Degrees Fluid capacity 7.6 pints wheel bearing Tapered Roller Tie Rod type Ball and socket Tie rod diameter 1.25" Tie rod thickness 0.22" Ball centers 60.15" housing pressed tubes wall thickness .50" | Dana 60 Application: 2500 HD, 2500 HD 4x4, 2500, 2500 4x4 Manual Transmission w/V8 engines. Capacity: 6,200 lbs type Full-floating Ratios 3.54, 4.09 Ring gear diameter 9.75" Ring gear face width N/A pinion mounting overhung pinion shaft diameter 1.63" inner 1.25" outer Differential pinions two axle shaft material Induction Hardened diameter at bearing none axle spline pitch 1.31" Spring Centers 44.50" Fluid capacity (2/4WD) 6.3/7.3 pints wheel bearings Tapered Roller | Dana 70 Application: 2500 HD, 2500 HD 4x4, 8.0L V10 and Diesel w/Auto Transmission. Capacity: 6,500 lbs type Full-floating Ratios 3.54, 4.10 Ring gear diameter 10.5" Ring gear face width N/A pinion mounting overhung pinion shaft diameter 1.75" inner 1.25" outer Differential pinions two axle shaft material Induction Hardened diameter at bearing none axle spline pitch 1.375" Spring Centers 43.26" Fluid capacity (2/4WD) 7.0/7.8 pints wheel bearings Tapered Roller |
| --- | --- | --- |