REMOVAL

| 1. | Remove the inlet hose and outlet hose. |
| --- | --- |

| 2. | Remove connectors (thermoswitch, heater and water sensor connector). |
| --- | --- |

| 3. | Unscrew fixing bolt and nut in bracket, then remove fuel filter. You must be careful of fuel spillage from fuel filter.. Tightening torque Fuel filter fixing bolts and nut: 30-35N·m (305.9 - 356.9 kg·cm, 22.1 - 25.8 lbf·ft). |
| --- | --- |